2004 EIS Submittal Information and Tips

Special Notes for 2004

- Include HRVOC Emissions

## Under the authority of LAC 33:III.919.B.4, emissions of highly reactive volatile organic compounds (HRVOC) must be included in the 2004 emissions inventory if the facility is located in one of the following 12 parishes:

| ## Ascension | ## Livingston | ## St. James |
| --- | --- | --- |
| ## East Baton Rouge | ## Pointe Coupee | ## St. John |
| ## East Feliciana | ## St. Charles | ## West Baton Rouge |
| ## Iberville | ## St. Helena | ## West Feliciana |

## If the facility is located in one of these 12 parishes, HRVOC emissions must be included in the electronic inventory at the point level and summarized on the Certification Statement.  The specific compounds that must be added and their SAROAD codes are:

| 1,3-butadiene | 43218 | Propylene | 43205 |
| --- | --- | --- | --- |
| Butenes, isomers | 43213 | Toluene* | 45202 |
| Ethylene | 43203 | Xylenes, isomers* | 45102 |

### *New for 2004

- Calculation of Emissions

## As per LAC 33:III.919.C, if continuous emissions monitoring system (CEMS) data or other stack test data are available, they must be used to calculate the emissions.  In lieu of test data, emissions shall be calculated using the best available information, i.e. emissions factors found in the most recent version of the Compilation of Air Pollution Emissions Factors (AP-42) or other approved estimation methodologies.  The most current estimation methodology shall be used, regardless of the methodology used in the permit.

Per Emergency Rule AQ240E1, effective December 28, 2004, emissions increases due solely to a change in AP-42 emission factors do not constitute violations of the air permit.  If the emission factors used in preparing the 2004 EIS data differ from the emission factors used in the current air permit(s) such that the resulting “calculated” emissions reflect an increase, please note the change in emission factor on the cover letter, the certification statement, and on card 30.  A copy of the cover letter and Certification Statement should be sent to the Office of Environmental Compliance, Enforcement Division, and the Office of Environmental Services, Permits: Title V & General Air Division.  In the cover letter accompanying the submittal, specify for each affected increase: the NEDS ID, SCC, new emission factor, & emission factor used in the current air permit.  The cover letter should also include the old and new emission factor reference source, date, volume and edition (if applicable), raw data used for that source category calculation, and any other explanation as described above as well as the facility’s intended time-frame to reconcile the emissions limits in the applicable permit(s).  A brief corresponding explanation should be included on card 30 of the EIS data file.

- Standard Oil and Gas Permits

## Facilities that operate under a Standard Oil & Gas Permit (SOGA) and are located in an ozone non-attainment parish or an adjoining parish must report EIS data, including Calcasieu, Beauregard, Cameron, and Jefferson Davis parishes.  The SOGA permit allows emissions above the reporting thresholds for these parishes.

- Title V Permits

## Facilities that operate under Title V Permits must report EIS data as per LAC 33:III.919.A.5.

- Email

Provide email address of facility contact on the Certification Statement.

- Data Accuracy

Data accuracy is vital!  It is the facility’s responsibility to provide good, quality data.  Files sent with too many errors will be sent back for corrections.

- Card 23 – Columns for Pollutant Codes must not be zero-filled

If a program is used that automatically adds zeros to columns 49-53, delete them before submitting the file.

REMINDERS:

- Consolidated Emissions Reporting Rule (FR Vol. 67, No. 111, pp 39602-39616, 6/10/02)

Requires reporting of the regional haze pollutants PM 2.5 and ammonia

PM 2.5 	Add to EIS data file under SAROAD Code 81104

Reference:	EPA’s PM 2.5 Inventory Resources Center

http://www.epa.gov/ttn/chief/eiip/pm25inventory/

Ammonia  	All facilities shall report all ammonia emissions in the EIS, even if

reported in TEDI.  Report under SAROAD Code 42604.

- Permit Information

All permit information fields in the electronic data, must be completed.  This includes:

Card 4 cols. 19-31 = permit number (if multiple, list the consolidated permit or “multiple”)

Card 13 cols. 26-32 = permitted emissions rate (lbs/hr)

Card 13 cols. 56-62 = annual permitted emissions (tons/yr)

Card 14 cols. 23-27 = permit emission point id

Card 14 cols. 29-72 = emission point description – should include sizes, capacities, fuels

and contents (ex. 1600 HP Nat Gas Compressor; 60,000 bbl Gasoline Tank)

- EIS and TEDI data files must correspond.

NEDS emission point IDs in the EIS and TEDI must represent the same process or equipment.

The toxic VOC total reported on the EIS Certification Statement must equal (within a rounding error) the toxic VOC total in the TEDI submittal.

- The data in the electronic file is used extensively for modeling purposes.

Data accuracy is vital!  Specifically, UTM coordinates and annual throughput on Card 11 and stack parameters on Card 12 must be correct.  Please verify the data in the submittal file.  Stack parameters must be entered for all emissions points unless the point is an area source or emitting at ground level (ex. truck cleaning, dust from unpaved roads, small heaters).  For sources that do not have a defined stack, enter the parameters applicable to the equipment (ex. enter a tank’s height and diameter).

POLICIES:

- Do not delete any Card 11s (“11D”).  This will result in deleting the entire NEDS point.  If a point has become obsolete:

- zero the throughput - Card 11 cols. 52-64 = 2525252500052

- zero all of the emissions - Card 13 cols. 26-62

- change the estimation code to 7 - Card 13 col. 64

- insert appropriate comment on Card 30

- The confidentiality code on Card 22 (col. 31) does not ensure data confidentiality.  Please see LAC 33:I.Chapter 5 for the DEQ procedure to request data confidentiality.

COMMON ERRORS TO AVOID:

- Code addresses correctly.  The facility’s physical address is on Card 02 (P.O. Boxes are not acceptable on this line).  The company’s mailing address is on Card 03.  Be sure to use 5 properly placed underscores.

- Pollutant emissions appear on Card 13.  Common errors are incorrect pollutant codes, numbers in the wrong columns and using decimal points.  There are no decimal points in the emissions estimates.  All emissions must be rounded to the nearest whole ton.  The exception is the ozone season day emissions, which have two significant digits beyond the decimal point.

- All particulate emissions must be reported as PM10 and PM2.5.  The SAROAD code for PM10 is 81102.  The SAROAD code for PM2.5 is 81104.  To change the electronic file from reporting TSP (11101) to PM10 (81102), use a Card 13 and 23 Delete for 11101 and a Card 13 and 23 Add for 81102.  To add PM2.5 (81104), use a Card 13 and 23 Add.

- Do not use an estimation code of 3 in column 64 of card 13.  Some commercially designed EIS software programs use the 3 in column 64.  Change the 3 to a valid code such as 5.

- The EIS data structure does not allow more than 16 pollutants to be reported per NEDS point.  Speciation of VOCs is not required, with the exception of HRVOC in the required parishes.  Report only the required pollutants.  They are:

| Pollutant | SAROAD Code |  | Optional (Greenhouse Gases) | SAROAD Code |
| --- | --- | --- | --- | --- |
| Ammonia (NH3) | 42604 |  | Carbon Dioxide (CO2) | 42102 |
| Carbon Monoxide (CO) | 42101 |  | Methane (CH4) | 43201 |
| Lead (Pb) | 12128 |  |  |  |
| Nitrogen Oxides (NOx) | 42602 |  |  |  |
| Particulate Matter 10 (PM10) | 81102 |  |  |  |
| Particulate Matter 2.5 (PM2.5) | 81104 |  |  |  |
| Sulfur Dioxide (SO2) | 42401 |  |  |  |
| Total Volatile Organic Compounds (VOC) | 43104 |  |  |  |
| Highly Reactive VOC* | SAROAD Code |  |  |  |
| 1,3-butadiene | 43218 |  |  |  |
| Butenes, isomers | 43213 |  |  |  |
| Ethylene | 43203 |  |  |  |
| Propylene | 43205 |  |  |  |
| Toluene* | 45202 |  |  |  |
| Xylenes, isomers* | 45102 |  |  |  |

*New for 2004

The total emissions reported under the VOC SAROAD code 43104 in the electronic file must equal the total VOCs (toxic plus non-toxic) reported on the Certification Statement, even if the VOCs are speciated in the electronic file.

- Do not use an action code of “C” when changing an SCC code on Cards 21-25.  To change an SCC code, delete the old one (use action code “D” on Card 21) and add the new one (use action code “A” for cards 21-25).  The SCC Scout program is available on the EIU Website for reference.

- When deleting an SCC record, use action code “D” on Card 21 and do not include cards 22, 23, 24, or 25 for the SCC being deleted.  Remove them from the electronic file completely.  Card 21 with an action code of “D” will remove all cards (21, 22, 23, 24, & 25) associated with the SCC record being deleted.

- If no change is being made to a card, no action code is needed. Leave column 80 blank.  If an action code is needed, use a capital letter.

| Parish Attainment Status |  |  |  |
| --- | --- | --- | --- |
| For the purposes of determining EIS applicability, see LAC 33:III.919.A |  |  |  |
| The following is the ozone attainment status for Louisiana Parishes. |  |  |  |
| Report at Non-Attainment Thresholds – LAC 33:III.919.A.1 |  |  |  |
| 1 Hour Ozone Non-Attainment Parishes |  | Attainment Parish with Activated Contingency Measures |  |
| Ascension |  | Calcasieu |  |
| East Baton Rouge |  |  |  |
| Iberville |  |  |  |
| Livingston |  |  |  |
| West Baton Rouge |  |  |  |
| Report at Adjoining Thresholds – LAC 33:III.919.A.2 |  |  |  |
| Adjoining Parishes |  | Parishes that adjoin a Parish w/Activated Contingency Measures |  |
| Assumption | St. John the Baptist | Beauregard |  |
| East Feliciana | St. James | Cameron |  |
| Iberia | St. Martin | Jefferson Davis |  |
| Pointe Coupee | Tangipahoa |  |  |
| St. Helena | West Feliciana |  |  |

For more information, please reference the EIS Coding Manual and the EIS Report Formatting Examples found on the EIU Website or call the EIU at (225) 219-3486.

EIU Website:		http://www.deq.louisiana.gov/evaluation/eis/index.htm