Title 33

ENVIRONMENTAL QUALITY

Part III.  Air

Chapter 5.  Permit Procedures

§509.  Prevention of Significant Deterioration

A. - I.4.  …

5.	The administrative authority may exempt a stationary source or modification from the requirements of Subsection M of this Section, with respect to monitoring for a particular pollutant, if:

a.	the emissions increase of the pollutant from a new stationary source or the net emissions increase of the pollutant from a modification would cause, in any area, air quality impacts less than the following amounts:

| Carbon monoxide | 575 µg/m3 | 8-hour average |
| --- | --- | --- |
| Nitrogen dioxide | 14 µg/m3 | annual average |
| Particulate matter | 10 µg/m3 of PM10 40 µg/m3 of PM2.51 | 24-hour average 24-hour average |
| Sulfur dioxide | 13 µg/m3 | 24-hour average |
| Ozone | No de minimis air quality level is provided for ozone. However, any net increase of 100 tons per year or more of volatile organic compounds or nitrogen oxides subject to PSD would require the performance of an ambient impact analysis including the gathering of ambient air quality data. |  |
| Lead | 0.1 µg/m3 | 3-month average |
| Fluorides | 0.25 µg/m3 | 24-hour average |
| Total reduced sulfur | 10 µg/m3 | 1-hour average |
| Hydrogen sulfide | 0.2 µg/m3 | 1-hour average |
| Reduced sulfur compounds | 10 µg/m3 | 1-hour average |

1 No exemption is available with regard to PM2.5.

I.5.b. - AA.15.b.  …

AUTHORITY NOTE:		Promulgated in accordance with R.S. 30:2054.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Air Quality and Nuclear Energy, Air Quality Division, LR 13:741 (December 1987), amended LR 14:348 (June 1988), LR 16:613 (July 1990), amended by the Office of Air Quality and Radiation Protection, Air Quality Division, LR 17:478 (May 1991), LR 21:170 (February 1995), LR 22:339 (May 1996), LR 23:1677 (December 1997), LR 24:654 (April 1998), LR 24:1284 (July 1998), repromulgated LR 25:259 (February 1999), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2447 (November 2000), LR 27:2234 (December 2001), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2437 (October 2005), LR 31:3135, 3156 (December 2005), LR 32:1600 (September 2006), LR 32:1843 (October 2006), LR 36:2556 (November 2010), LR 37:1148 (April 2011), repromulgated LR 37:1389 (May 2011), amended LR 37:1570 (June 2011), repromulgated LR 37:2146 (July 2011), amended by the Office of the Secretary, Legal Division, LR 38:3163 (December 2012), LR 39:1280 (May 2013), LR 42:**.