Title 33

ENVIRONMENTAL QUALITY

Part III.  Air

Chapter 5.  Permit Procedures

§504.  Nonattainment New Source Review (NNSR) Procedures and Offset Requirements in Specified Parishes

A. - A.2.  …

3.	The emissions increase that would result from a proposed modification, without regard to project decreases, shall be compared to the trigger values listed in Subsection L, Table 1 of this Section to determine whether a calculation of the net emissions increase over the contemporaneous period must be performed

A.3.a. - K.Visibility Impairment.  …

L. 	Table 1—Major Stationary Source/Major Modification Emission Thresholds

| Table 1 Major Stationary Source/Major Modification Emission Thresholds |  |  |  |
| --- | --- | --- | --- |
| Pollutant | Major Stationary Source Threshold Values (tons/year) | Major Modification Significant Net Increase (tons/year) | Offset Ratio Minimum |
| Ozone VOC/NOx |  | Trigger Values |  |
| Marginal | 100 | 40(40)2 | 1.10 to 1 |
| Moderate | 100 | 40(40)2 | 1.15 to 1 |
| Serious | 50 | 253(5)4 | 1.20 to 1 w/LAER or 1.40 to 1 internal w/o LAER |
| Severe | 25 | 253(5)4 | 1.30 to 1 w/LAER or 1.50 to 1 internal w/o LAER |
| Extreme | 10 | Any increase | 1.50 to 1 |
| CO |  |  |  |
| Moderate | 100 | 100 | >1.00 to 1 |
| Serious | 50 | 50 | >1.00 to 1 |
| SO2 | 100 | 40 | >1.00 to 1 |
| PM101 |  |  |  |
| Moderate | 100 | 15 | >1.00 to 1 |
| Serious | 70 | 15 | >1.00 to 1 |
| PM2.55 | 100 | 10 | >1.00 to 1 |
| Lead | 100 | 0.6 | >1.00 to 1 |

Footnote 1.  …

2 	Consideration of the net emissions increase will be triggered for any project that would increase emissions by 40 tons or more per year, without regard to any project decreases.

Footnote 3.  …

4 	Consideration of the net emissions increase will be triggered for any project that would increase VOC or NOX emissions by 5 tons or more per year, without regard to any project decreases, or for any project that would result in a 25 ton or more per year cumulative increase in emissions of VOC or NOX within the contemporaneous period or of NOx for a period of five years after the effective date of the rescission of the NOx waiver, and within the contemporaneous period thereafter.

Footnote 5.  …

M. 	Offset Requirements in Specified Parishes. Except as provided in Paragraph M.4 of this Section, the provisions of this Subsection shall apply to stationary sources located in the parishes of Ascension, East Baton Rouge, Iberville, Livingston, and West Baton Rouge if the parish’s designation with respect to the 8-hour national ambient air quality standard (NAAQS) for ozone is attainment, marginal nonattainment, or moderate nonattainment.

1.  …

2.	Existing Stationary Sources

a.	Consideration of the net emissions increase shall be triggered for any physical change or change in the method of operation that would increase emissions of VOC or NOX by 40 tons per year or more, without regard to any project decreases.

M.2.b. - M.5.b.  …

6.	Creditable emission reductions from sources located in Livingston Parish achieved prior to the removal of Livingston Parish from the list of affected parishes in LAC 33:III.504.M (and for which an ERC application has been submitted in accordance with LAC 33:III.615) shall remain eligible for use as offsets. However, emission reductions from sources located in Livingston Parish realized after Livingston Parish has been removed from the aforementioned list of affected parishes shall no longer be eligible for use as offsets for purposes of LAC 33:III.504.M.

AUTHORITY NOTE: 	Promulgated in accordance with R.S. 30:2054.

HISTORICAL NOTE: 	Promulgated by the Department of Environmental Quality, Office of Air Quality and Radiation Protection, Air Quality Division, LR 19:176 (February 1993), repromulgated LR 19:486 (April 1993), amended LR 19:1420 (November 1993), LR 21:1332 (December 1995), LR 23:197 (February 1997), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2445 (November 2000), LR 27:2225 (December 2001), LR 30:752 (April 2004), amended by the Office of Environmental Assessment, LR 30:2801 (December 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2436 (October 2005), LR 31:3123, 3155 (December 2005), LR 32:1599 (September 2006), LR 33:2082 (October 2007), LR 34:1890 (September 2008), LR 37:1568 (June 2011), LR 38:1232 (May 2012), amended by the Office of the Secretary, Legal Division, LR 38:2766 (November 2012), LR 41:2135 (October 2015), LR 42:403 (March 2016), LR 43:926 (May 2017), LR 45