Title 33

ENVIRONMENTAL QUALITY

Part VII.  Solid Waste

Subpart 1.  Solid Waste Regulations

Chapter 7.	Solid Waste Standards

Subchapter A.	Landfills, Surface Impoundments, and/or Landfarms

§711.	Standards Governing Landfills (Type I and II)

A. — B.4.f.vii.	…

viii.	The leachate head shall be maintained in a pumped-down condition such that not more than 1 foot of head shall exist above the lowest elevation of the leachate collection lines. Compliance with the 1 foot leachate head standard shall be achieved by compliance with LAC 33:VII.711.D.3.e. and f, and the provisions of the facility operational plan that address LAC 33:VII.711.D.2.g.-i.

B.4.f.ix. — C.2.c.xi.	…

xii.	records on the leachate head, volume(s), and results of the leachate sampling, if applicable;

xiii. — xv.	…

xvi.	post-closure monitoring reports; and

xvii.	copies of all documents received from and submitted to the department.;

xviii.	records on leachate pump tests; and

xix.	records of action(s) taken under LAC 33:VII.711.D.2.i.

C.3. — D.2.f.	…

g.	the methods that will be used to ensure that the leachate collection/treatment system is functioning as designed; including, but not limited to:

i.	the testing and inspection of system components;

ii.	the repair and/or replacement of system components; and

iii.	the time frame for such repair and/or replacements;and

h.	the measuring protocol, frequency, and recordkeeping to be used to monitor leachate head and the frequency with which the depth of leachate within the collection system will be checked, as well as how the leachate will be removed and transported to the treatment facility.; and

i.	the actions to be taken when monitoring indicates leachate head exceeds the standard in LAC 33:VII.711.B.4.f.viii.

3. — 3.d.	…

e.	Leachate Pumps

i.	Leachate pumps shall be tested at least weekly, maintained, and operated to ensure compliance with LAC 33:VII.711.B.4.f.viii.

ii.	Testing results shall be documented within 24 hours and maintained in accordance with LAC 33:VII.711.C.2.a-c.

iii.	For nonfunctioning leachate pumps and/or not maintaining liquid head below the 1 foot leachate head standard set forth in LAC 33:VII.711.B.4.f.viii, action to repair and/or to replace pumps shall be initiated and completed within seven days, or as otherwise approved by the administrative authority.

iv.	Any request for an extension under LAC 33:VIII.711.D.3.iii shall be submitted for approval to the Office of Environmental Services and shall at a minimum  include:

(a).	reason(s) for the extension; and

(b).	estimated time for repair or replacement, including supporting documentation.

f.	Leachate Head Monitoring

i.	Leachate head shall be monitored and recorded:

(a).	in accordance with the facility operational plan at least every normal operating day; or

(i).	as otherwise approved by the administrative authority for cells without final cover; and

(b).	at least every seven days for cells that have received final cover.

ii.	If monitoring of leachate head shows an exceedance of more than 1 foot of head above the lowest elevation of the leachate collection lines under the facility operational plan, action shall be taken in accordance with LAC 33:VII.711.D.2.i.

D.4. — F.3.d.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 19:1143 (September 1993), repromulgated LR 19:1316 (October 1993), amended by the Office of the Secretary, LR 24:2251 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2523 (November 2000), repromulgated LR 27:704 (May 2001), amended LR 30:1676 (August 2004), amended by the Office of Environmental Assessment, LR 30:2024 (September 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2492 (October 2005), LR 33:1047 (June 2007), LR 33:2145 (October 2007), LR 34:1901 (September 2008), LR 37:1564 (June 2011), LR 37:3248 (November 2011), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division LR 49: