NOTICE OF INTENT

Department of Environmental Quality

Office of the Secretary

Legal Affairs and Criminal Investigations Division

Revision to Update Solid Waste Regulations

(LAC 33:VII.115, 721, 10505, 10509, 10523 and 10529)

Under the authority of the Environmental Quality Act, R.S. 30:2001 et seq., and in accordance with the provisions of the Administrative Procedure Act, R.S. 49:950 et seq., the secretary gives notice that rulemaking procedures have been initiated to amend the  regulations, LAC 33:VII.115, 721, 10505, 10509, 10519, 10523 and 10529 (SW075).

This Rule provides updates to facilitate implementation of the legislative mandate of remediating environmental and public health problems caused by improper disposal of waste tires. These proposed regulations establish procedures, standards, and requirements for Solid Waste Type III landfills to collect, transport, prepare for disposal, and properly dispose of waste tires. The adoption of these regulations would assist in addressing the increasing problem of promiscuous tire dumping in the state of Louisiana. The basis and rationale for this Rule are to revise and update the Solid Waste regulations to ensure proper collection, transportation,and disposal of waste tires and disposable waste tire material. This Rule meets an exception listed in R.S. 30:2019(D)(2) and R.S. 49:963.B(3); therefore, no report regarding environmental/health benefits and social/economic costs is required.

Family Impact Statement

This Rule has no known impact on family formation, stability, and autonomy as described in R.S. 49:972.

Poverty Impact Statement

This Rule has no known impact on poverty as described in R.S. 49:973.

Small Business Analysis

This Rule has no known impact on small business as described in R.S. 49:974.1 - 974.8.

Provider Impact Statement

This Rule has no known impact on providers as described in HCR 170 of 2014.

Public Comments

All interested persons are invited to submit written comments on the proposed regulation. Persons commenting should reference this proposed regulation by SW075. Such comments must be received no later than September 5, 2023, at 4:30 p.m., and should be sent to William Little, Attorney Supervisor, Office of the Secretary, Legal Affairs Division, P.O. Box 4302, Baton Rouge, LA 70821-4302, by fax (225) 219-4068, or by E-mail to DEQ.Reg.Dev.Comments@la.gov. Copies of these proposed regulations can be purchased by contacting the DEQ Public Records Center at (225) 219-3168. Check or money order is required in advance for each copy of SW075. These proposed regulations are available on the Internet at www.deq.louisiana.gov/portal/tabid/1669/default.aspx.

Public Hearing

A public hearing will be held via Zoom on August 29, 2023, at 1:30 p.m. Interested persons are invited to attend and submit oral comments via PC, Mac, Linux, iOS or Android at https://deqlouisiana.zoom.us/j/88185532848?pwd=RWRRZ3Z0aURtdVdqWStmZGlRTnRYQT09, password 083515 or by telephone by dialing 636-651-3182 using the conference code 725573. Should individuals with a disability need an accommodation in order to participate, contact William Little at the address given below or at (225) 219-3985.

These proposed regulations are available for inspection at the following DEQ office locations from 8 a.m. until 4:30 p.m.:  602 N. Fifth Street, Baton Rouge, LA 70802; 1823 Highway 546, West Monroe, LA 71292; State Office Building, 1525 Fairfield Avenue, Shreveport, LA 71101; 1301 Gadwall Street, Lake Charles, LA 70615; 111 New Center Drive, Lafayette, LA 70508; 110 Barataria Street, Lockport, LA 70374; 201 Evans Road, Bldg. 4, Suite 420, New Orleans, LA  70123.

Courtney J. Burdette

Executive Counsel

ENVIRONMENTAL QUALITY

Part VII.  Solid Waste

Subpart 1.  Solid Waste Regulations

Chapter 1.	General Provisions and Definitions

§115.	Definitions

A.	…

* * *

Disposable Tire Material—material produced from whole waste tires which have been cut, chipped, shredded, or prepared by other volume reduction methods, as approved by the administrative authority, to facilitate disposal.

* * *

Type III Facility—a facility used for disposing or processing of construction/demolition debris, disposable tire material, or woodwaste, or composting organic waste to produce a usable material, or preparing whole waste tires for disposal, or separating recyclable wastes (e.g., a construction/demolition-debris or woodwaste landfill, separation facility, or composting facility).

* * *

Waste Tire—a whole tire that is no longer suitable for its original purpose because of wear, damage, or defect and/or has been discarded by the consumer.

* * *

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq. and in particular R.S. 30:2154.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 22:279 (April 1996), amended by the Office of Waste Services, Solid Waste Division, LR 23:1145 (September 1997), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2514, 2609 (November 2000), amended by the Office of Environmental Assessment, LR 31:1576 (July 2005), amended by the Office of the Secretary, Legal Affairs Division, LR 33:1019 (June 2007), LR 34:1023 (June 2008), LR 34:1399 (July 2008), LR 37:1563 (June 2011), LR 37:3233 (November 2011), LR 38:46 (January 2012), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 45:234 (February 2019), LR 49:

Chapter 7.	Solid Waste Standards

Subchapter C.	Minor Processing and Disposal Facilities

§721.	Standards Governing Construction and Demolition Debris and Woodwaste Landfills (Type III)

A. — B.2.c.v.	…

d.	The permit holder who completes waste tire generator and/or processor manifests must comply with LAC 33:VII.10534.

B.3. — C.1.e.i.	…

ii.	woodwaste, as defined in LAC 33:VII.115; and

iii.	yard trash, as defined in LAC 33:VII.115;. and

iv.	disposable tire material, as defined in LAC 33:VII.115.

f. — h.	…

i.	No more than 150 waste tires shall be stored at one time, unless stored in a container in accordance with LAC 33:VII.503.A.2, or as approved by the administrative authority.

j.	Disposable tire material shall not comprise more than 50 percent of the volume of the waste disposed during any 14-day period.

k.	Placement of disposable tire material in any contiguous area shall be no more than 4 feet in depth prior to application of cover or other waste types sufficient to serve as a fire break.

l.	Neither whole waste tires nor disposable tire material shall be stored for longer than 365 days prior to disposal.

2. — 2.g.	…

h. 	facilities that dispose of disposable tire material shall:

i.	enter into a written agreement with the local fire department regarding fire protection at the facility;

ii.	develop and implement a fire protection and safety plan for the facility to ensure personnel protection and minimize impact to the environment; and

iii.	develop a waste tire volume reduction and disposal plan.

C.3. — E.3.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 20:1001 (September 1994), amended by the Office of the Secretary, LR 24:2252 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2527 (November 2000), repromulgated LR 27:705 (May 2001), amended by the Office of Environmental Assessment, LR 30:2025 (September 2004), LR 31:1577 (July 2005), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2495 (October 2005), LR 33:1067 (June 2007), LR 33:2149 (October 2007), LR 34:1901 (September 2008), LR 37:1566 (June 2011), LR 37:3253 (November 2011), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 49:

Subpart 2.  Recycling

Chapter 105	Waste Tires

§10505.	Definitions

A.	…

* * *

Government Tire Sweep—a waste tire collection event authorized by the administrative authority to allow government agencies to collect waste tires for transport to a permitted waste tire processing facility or permitted solid waste landfill.

* * *

Processing—any method or activity that alters whole waste tires so that they are no longer whole; such as, cutting, slicing, chipping, shredding, distilling, freezing, or other processes as determined by the administrative authority. At a minimum, a tire is considered processed only if its volume has been reduced by more than half. For the purposes of this Chapter 105, processing does not include the preparation of waste tires or waste tire material into disposable tire material as defined at LAC 33:VII.115 for disposal in a permitted landfill.

* * *

Program Ineligible Waste Tire—a waste tire for which a processor will not be reimbursed from the waste tire management fund. This includes, but is not limited to, tires weighing 500 pounds or more at the time of sale, solid tires, tires purchased from a tire wholesaler for use on fleet vehicles and/or used vehicles for which a fee has not been paid, out-of-state tires, marine bumper tires, purchased used tires that are not suitable for re-sale, tires accepted by retail outlets for which a fee has not been collected, tires converted to disposable tire material and disposed of at a permitted solid waste landfill, and any other tire not defined as a program eligible waste tire.

* * *

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2411-2422.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 18:37 (January 1992), amended LR 20:1001 (September 1994), LR 22:1213 (December 1996), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2773 (December 2000), LR 27:829 (June 2001), LR 27:2226 (December 2001), LR 28:1953 (September 2002), LR 29:2779 (December 2003), amended by the Office of Environmental Assessment, LR 31:1323 (June 2005), amended by the Office of the Secretary, Legal Affairs Division, LR 31:3158 (December 2005), LR 33:89 (January 2007), amended by the Office of the Secretary, Legal Division, LR 42:244 (February 2016), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 49:

§10509.	Prohibitions and Mandatory Provisions

A.	No person shall knowingly and/or intentionally dispose unprocessedof whole waste tires in a landfill within the boundaries of the state of Louisiana. Any landfill disposal of waste tire material must be at a facility permitted to receive waste tires or disposable tire material in accordance with LAC 33:VII.Chapter 7.

B. — C.1.	…

2.	collected and stored at an authorized waste tire transfer station, authorized waste tire collection center, permitted solid waste landfill, or permitted waste tire processing facility;

C3. — D.	…

E.	No processor shall receive payment from the Waste Tire Management Fund without a standard processing permit issued by the administrative authority in accordance with §10511, §10513, and §10517 of this Chapter, an effective Processor’s Agreement, and an approved end-market use project in which whole waste tires and/or waste tire material are utilized.

F. — J.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2411-2422.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 18:38 (January 1992), amended LR 20:1001 (September 1994), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2774 (December 2000), amended by the Office of Environmental Assessment, LR 31:1323 (June 2005), amended by the Office of the Secretary, Legal Division, LR 42:248 (February 2016) ,  amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 49:

§10523.	Standards and Responsibilities of Waste Tire Transporters

A. — F.	…

G.	For in-state waste tire transportation, the transporter shall transport all waste tires only to an authorized collection center, an authorized waste tire transfer station, a permitted processing facility, a permitted solid waste landfill, or an authorized end-market use.

H. — K.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2411 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 18:41 (January 1992), amended LR 20:1001 (September 1994), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2778 (December 2000), LR 27:831 (June 2001), repromulgated LR 27:1885 (November 2001), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2503 (October 2005), LR 33:2159 (October 2007), amended by the Office of the Secretary, Legal Division, LR 42:257 (February 2016), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 49:

§10529.	Standards and Procedures for Waste Tire Cleanups

A. — B.1.l.	…

2.	All waste tires collected shall be removed by an authorized waste tire transporter and processed by the permitted waste tire processor or transported to a permitted solid waste landfill indicated on the single event cleanup/government tire sweep form submitted to the administrative authority. Use of a waste tire processor not indicated on the form must be approved in writing by the administrative authority.

B.3. — D.8.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2411 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 20:1001 (September 1994), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2780 (December 2000), amended by the Office of the Secretary, Legal Division, LR 42:262 (February 2016), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 49:

FISCAL AND ECONOMIC IMPACT STATEMENT

FOR ADMINISTRATIVE RULES

Person

Preparing

Statement:	Jodie Kimball                     	Dept.:	Environmental Quality

Phone:	(225) 219-3045                   	Office:	Waste Permit Division

Return		Rule

Address:	602 N. 5th Street           	Title:	Revision to Update Solid Waste

Regulations (LAC 33:VII.115, 721,

Baton Rouge, LA 70802     		10505, 10509, 10523, and 10529)

Date Rule

Takes Effect:  Upon promulgation

SUMMARY

(Use complete sentences)

In accordance with Section 961 of Title 49 of the Louisiana Revised Statutes, there is hereby submitted a fiscal and economic impact statement on the rule proposed for adoption, repeal or amendment. THE FOLLOWING STATEMENTS SUMMARIZE ATTACHED WORKSHEETS, I THROUGH IV AND WILL BE PUBLISHED IN THE LOUISIANA REGISTER WITH THE PROPOSED AGENCY RULE.

I.	ESTIMATED IMPLEMENTATION COSTS (SAVINGS) TO STATE OR LOCAL GOVERNMENTAL 	UNITS (Summary)

There are no estimated implementation costs or savings to state governmental units as a result of the proposed rule change.

The proposed rule provides for revisions to the solid waste regulations to ensure proper collection,  disposal of waste tires and disposable tire material, and provides for the updates authorized in        Act 291 of the 2021 Regular Legislative Session. Also, the proposed rule change allows Solid Waste Type III landfill operators to collect, process, and properly dispose of waste tires.

II. 	ESTIMATED EFFECT ON REVENUE COLLECTIONS OF STATE OR LOCAL GOVERNMENTAL 	UNITS (Summary)

There is no anticipated effect on revenue collections of state and local governmental units from this proposed action.

III. 	ESTIMATED COSTS AND/OR ECONOMIC BENEFITS TO DIRECTLY AFFECTED PERSONS, SMALL BUSINESSES, OR NON-GOVERNMENTAL GROUPS (Summary)

The proposed rule change will not result in increased costs to anyone purchasing a tire.  The change would allow for Solid Waste Type III operators to landfill disposable tire material.  This rule change would provide an additional revenue stream should the operators take advantage of it. Operators may experience a one-time cost for the purchase of equipment to process and dispose waste tires.

IV. 	ESTIMATED EFFECT ON COMPETITION AND EMPLOYMENT (Summary)

The proposed rule change would increase competition for sourcing waste tires from waste generators and one-time cleanups.  If Type III operators chose to invest in equipment to process whole waste tires and to the extent the workload significantly increases, there may be an increase in employment opportunities.

Signature of Agency Head or Designee				Legislative Fiscal Officer or Designee

Courtney J. Burdette, Executive Counsel

Typed Name & Title of Agency Head or Designee

Date of Signature                                  		Date of Signature

FISCAL AND ECONOMIC IMPACT STATEMENT

FOR ADMINISTRATIVE RULES

The following information is required in order to assist the Legislative Fiscal Office in its review of the fiscal and economic impact statement and to assist the appropriate legislative oversight subcommittee in its deliberation on the proposed rule.

- Provide a brief summary of the content of the rule (if proposed for adoption, or repeal) or a brief summary of the change in the rule (if proposed for amendment). Attach a copy of the notice of intent and a copy of the rule proposed for initial adoption or repeal (or, in the case of a rule change, copies of both the current and proposed rules with amended portions indicated).

The proposed rule provides for revisions to the solid waste regulations to ensure proper collection, disposal of waste tires and disposable tire material, and provides for the updates authorized in Act 291 of the 2021 Regular Legislative Session.

The proposed rule change allows Solid Waste Type III landfill operators to collect, transport, process, properly dispose of waste tires.

- Summarize the circumstances, which require this action. If the Action is required by federal regulation, attach a copy of the applicable regulation.

The proposed rule change revises and updates the solid waste regulations to ensure proper collection, disposal of waste tires and disposable tire material and provides for the updates authorized in Act 291 of the 2021 Regular Legislative Session.

- Compliance with Act 11 of the 1986 First Extraordinary Session

  - Will the proposed rule change result in any increase in the expenditure of funds? If so, specify amount and source of funding.

This proposed rule will not result in any increase in the expenditure of funds.

(2) 	If the answer to (1) above is yes, has the Legislature specifically appropriated the funds necessary for the associated expenditure increase?

(a)                 	Yes. If yes, attach documentation.

(b)                 	NO. If no, provide justification as to why this rule change should be published at this time

FISCAL AND ECONOMIC IMPACT STATEMENT

WORKSHEET

I.	A. 	COSTS OR SAVINGS TO STATE AGENCIES RESULTING FROM THE ACTION PROPOSED

- What is the anticipated increase (decrease) in costs to implement the proposed action?

COSTS	FY 24	FY 25	FY 26

Personal Services		             -0- 	 	 	      -0-			            -0-

Operating Expenses		             -0- 	 	 	      -0-			            -0-

Professional Services		             -0- 	 	 	      -0-			            -0-

Other Charges			             -0- 	 	 	      -0-			            -0-

Equipment			             -0- 	 	 	      -0-			            -0-

Major Repairs & Constr.		             -0- 	 	 	      -0-			            -0-

TOTAL				             -0- 	 	 	      -0-			            -0-

POSITIONS (#)

- Provide a narrative explanation of the costs or savings shown in "A. 1.", including the increase or reduction in workload or additional paperwork (number of new forms, additional documentation, etc.) anticipated as a result of the implementation of the proposed action. Describe all data, assumptions, and methods used in calculating these costs.

There is no anticipated increase or reduction in costs or savings as a result of the implementation of the proposed action.

- Sources of funding for implementing the proposed rule or rule change.

SOURCE	FY 24	FY 25	FY 26

State General Fund		             -0- 	 	 	      -0-			            -0-

Agency Self-Generated		             -0- 	 	 	      -0-			            -0-

Dedicated			             -0- 	 	 	      -0-			            -0-

Federal Funds			             -0- 	 	 	      -0-			            -0-

Other (Specify)			             -0- 	 	 	      -0-			            -0-

TOTAL				             -0- 	 	 	      -0-			            -0-

- Does your agency currently have sufficient funds to implement the proposed action? If not, how and when do you anticipate obtaining such funds?

No additional funding is required on the part of the agency to implement the proposed   action.

B.		COST OR SAVINGS TO LOCAL GOVERNMENTAL UNITS RESULTING FROM THE ACTION PROPOSED.

- Provide an estimate of the anticipated impact of the proposed action on local governmental units, including adjustments in workload and paperwork requirements.  Describe all data, assumptions and methods used in calculating this impact.

The proposed changes do not apply to local governmental units and will have no impact on them.

- Indicate the sources of funding of the local governmental unit, which will be affected by these costs or savings.

Local governmental units are not affected by the proposed action, so no sources of funding of local governmental units will be affected.

FISCAL AND ECONOMIC IMPACT STATEMENT

WORKSHEET

II.	EFFECT ON REVENUE COLLECTIONS OF STATE AND LOCAL GOVERNMENTAL UNITS

- What increase (decrease) in revenues can be anticipated from the proposed action?

REVENUE INCREASE/DECREASE	        FY 24	FY 25	FY 26

State General Fund 			      -0- 	 	 	      -0-			            -0-

Agency Self-Generated		                     -0- 	 	 	      -0-			            -0-

Dedicated Funds*			       -0- 	 	 	      -0-			            -0-

Federal Funds			          	       -0- 	 	 	      -0-			            -0-

Local Funds				       -0- 	 	 	      -0-			            -0-

TOTAL					       -0- 	 	 	      -0-			            -0-

*Specify the particular fund being impacted.

- Provide a narrative explanation of each increase or decrease in revenues shown in "A."  Describe all data, assumptions, and methods used in calculating these increases or decreases.

The proposed action would not result in any appreciable increase or decrease in state or local revenue collections.

FISCAL AND ECONOMIC IMPACT STATEMENT

WORKSHEET

III.	COSTS AND/OR ECONOMIC BENEFITS TO DIRECTLY AFFECTED PERSONS, SMALL BUSINESSES, OR NONGOVERNMENTAL GROUPS

A.	What persons, small businesses, or non-governmental groups would be directly affected by the proposed action? For each, provide an estimate and a narrative description of any effect on costs, including workload adjustments and additional paperwork (number of new forms, additional documentation, etc.), they may have to incur as a result of the proposed action.

The proposed rule change may impact Solid Waste Type III landfill operators should they chose to landfill disposable tire material which was previously not allowed. Operators may see an increase in revenues should they chose to dispose waste tires. Estimates of the number of facilities that will participate is unknown at this time. There are approximately 30 Solid Waste Type III facilities in the state.

If Type III operators chose to invest in equipment to process whole waste tires, they are likely to experience up-front costs for new equipment that is necessary to process waste tires. To the extent the workload significantly increases, there may be an increase in employment opportunities as well.

- Also provide an estimate and a narrative description of any impact on receipts and/or income resulting from this rule or rule change to these groups.

Landfill operators may experience an impact on receipts and/or income as they would have the ability to assess tipping fees for receiving whole waste tires and disposable tire materials for processing and disposal.

IV. 	EFFECTS ON COMPETITION AND EMPLOYMENT

Identify and provide estimates of the impact of the proposed action on competition and employment

in the public and private sectors. Include a summary of any data, assumptions and methods used in making these estimates.

The proposed rule change would increase competition for sourcing waste tires from waste generators and one-time cleanups.  If Type III operators chose to invest in equipment to process whole waste tires and to the extent the workload significantly increases, there may be an increase in employment opportunities.