Title 33

ENVIRONMENTAL QUALITY

Part V.  Hazardous Waste and Hazardous Materials

Subpart 1.  Department of Environmental Quality—Hazardous Waste

Chapter 1.	General Provisions and Definitions

§105.	Program Scope

These rules and regulations apply to owners and operators of all facilities that generate, transport, treat, store, or dispose of hazardous waste, except as specifically provided otherwise herein. The procedures of these regulations also apply to the denial of a permit for the active life of a hazardous waste management facility or individual unit at a treatment, storage, and disposal (TSD) facility under LAC 33:V.706. Definitions appropriate to these rules and regulations, including solid waste and hazardous waste, appear in LAC 33:V.109. Wastes that are excluded from regulation are found in this Section.

A.	…

1.	Within 90 days after the promulgation or revision of these regulations anyone subject to these regulations who has not previously notified the department on the Notification of Hazardous Waste Activity Form (HW-1), or whose notification on the HW-1 fForm is not approved, must shall notify the Office of Environmental Services, using the most current HW-1 fForm or other form approved by the department.

2. — 6.	…

a.	If rejected, the applicantperson shall resubmit the notification using the appropriate, approved form.

6.b. — 7.	…

8.	Facilities who cease hazardous waste activities shall notify the Office of Environmental Services within 30 days using the department’s Notification of Hazardous Waste Activity Form (HW-1) or other forms approved by the department in accordance with Subparagraph 105.A.5.b of this Section.Required Notifications for Ceasing Regulated Hazardous Waste Activities

a.	A large quantity generator closing a facility shall follow the closure notification requirements in LAC 33:V.1015.B.8 using the HW-1 Form. If no other regulated hazardous waste activities will be occurring at the facility, the EPA identification number for the facility will be deactivated once verified by the department.

b.	A generator ceasing an individual regulated hazardous waste activity when there is ongoing regulated hazardous waste activities, shall submit the HW-1 Form to notify the department within seven calendar days of ceasing the activity.) The EPA identification number for the facility will continue to be active for the ongoing regulated hazardous waste activities. (NOTE: LAC 33:V.105.A.8.b does not apply to a large quantity generator. A large quantity generator shall instead comply with the closure notification requirements in LAC 33:V.1015.B.8, as applicable.)

c.	A generator ceasing all regulated hazardous waste activities at a facility shall notify the department within 30 calendar days using the most current Certification of No Hazardous Waste Form (Form 7442), or other forms approved by the department. The EPA identification number for the facility will be deactivated once verified by the department. (NOTE: a large quantity generator shall also comply with the closure notification requirements in LAC 33:V.1015.B, as applicable.)

9.	Failure to submit a timely and complete Notification of Hazardous Waste Activity Form (HW-1), obtain an active EPA identification number or notify the department of changes to the notification shall constitute a violation of these regulations and subject the applicant to enforcement action up to and including the assessment of civil penalties.Moving Hazardous Waste Activities to Another Location

a.	A person moving a regulated hazardous waste activity to another location (i.e., physical address) shall notify the department within 30 days:

i.	by submitting the HW-1 Form to obtain a new EPA identification number for the regulated hazardous waste activity at the new physical address; and

ii.	if the regulated hazardous waste activity will no longer be conducted at the former facility, submit the required notifications for ceasing regulated hazardous waste activities as required by SubParagraph A.8.a.

10.	Failure to submit a timely and complete Notification of Hazardous Waste Activity Form (HW-1), obtain an active EPA identification number, or notify the department of changes to the notification shall constitute a violation of these regulations and the owner and/or operator shall be subject to an enforcement action up to and including the assessment of civil penalties.

B. — D.1.f.	…

g.	spent sulfuric acid used to produce virgin sulfuric acid provided it is not accumulated speculatively as defined in LAC 33:V.109. Solid Waste;

D.1.h. — R.8.h.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq., and in particular, 2186(A)(2).

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 10:200 (March 1984), amended LR 10:496 (July 1984), LR 11:1139 (December 1985), LR 12:319 (May 1986), LR 13:84 (February 1987), LR 13:433 (August 1987), LR 13:651 (November 1987), LR 14:790 (November 1988), LR 15:181 (March 1989), LR 16:47 (January 1990), LR 16:217, LR 16:220 (March 1990), LR 16:398 (May 1990), LR 16:614 (July 1990), LR 17:362, 368 (April 1991), LR 17:478 (May 1991), LR 17:883 (September 1991), LR 18:723 (July 1992), LR 18:1256 (November 1992), LR 18:1375 (December 1992), amended by the Office of the Secretary, LR 19:1022 (August 1993), amended by the Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 20:1000 (September 1994), LR 21:266 (March 1995), LR 21:944 (September 1995), LR 22:813, 831 (September 1996), amended by the Office of the Secretary, LR 23:298 (March 1997), amended by the Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 23:564, 567 (May 1997), LR 23:721 (June 1997), amended by the Office of Waste Services, Hazardous Waste Division, LR 23:952 (August 1997), LR 23:1511 (November 1997), LR 24:298 (February 1998), LR 24:655 (April 1998), LR 24:1093 (June 1998), LR 24:1687, 1759 (September 1998), LR 25:431 (March 1999), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:268 (February 2000), LR 26:2464 (November 2000), LR 27:291 (March 2001), LR 27:706 (May 2001), LR 29:317 (March 2003), LR 30:1680 (August 2004), amended by the Office of Environmental Assessment, LR 30:2463 (November 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2451 (October 2005), LR 32:605 (April 2006), LR 32:821 (May 2006), LR 33:450 (March 2007), LR 33:2097 (October 2007), LR 34:614 (April 2008), LR 34:1008 (June 2008), LR 34:1893 (September 2008), LR 34:2395 (November 2008), LR 35:1878 (September 2009), LR 36:2553 (November 2010), LR 38:791 (March 2012), amended by the Office of the Secretary, Legal Division. LR 40:1336 (July 2014), LR 42:2178, 2181 (December 2016), LR 43:1151 (June 2017), repromulgated by the Office of the Secretary, Legal Affairs and Criminal Investigation Division, LR 43:1523 (August 2017), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:896 (July 2020), LR 47:1851 (December 2021), amended by the Office of the Secretary, Legal Affairs Division LR 50:1456 (October 2024), LR 52:

§109.	Definitions

For all purposes of these rules and regulations, the terms defined in this Chapter shall have the following meanings, unless the context of use clearly indicates otherwise.

* * *

Final Closure—the closure of all hazardous waste management units at the facility in accordance with all applicable closure requirements so that hazardous waste management activities under LAC 33:V.Chapters 15, 19, 21, 23, 25, 27, 29, 31, 33, 35, and 43 are no longer conducted unless subject to provisions of LAC 33:V.1009, 1011, 1013, and 1015.

* * *

Large Quantity Generator—a generator who generates any of the following amounts in a calendar month:

1. — 2.	…

3.	greater than 100 kilograms (220 lbs) of any residue or contaminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute hazardous waste listed in LAC 33:V.4901.B with the assigned hazard code of (H) or LAC 33.V.4901.E.

* * *

Small Quantity Generator—a generator who generates the following amounts in a calendar month:

1. — 2.	…

3.	less than or equal to 100 kilograms (220 lbs) of any residue or contaminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute hazardous waste listed in LAC 33:V.4901.B with the assigned hazard code of (H) or LAC 33.V.4901.E.

* * *

Very Small Quantity Generator—a generator who generates less than or equal to the following amounts in a calendar month:

1.	…

2.	 1 kilogram (2.2 lbs) of acute hazardous waste listed in in LAC 33:V.4901.B with the assigned hazard code of (H) or LAC 33:V.4901.E; and

3.	100 kilograms (220 lbs) of any residue or contaminated soil, water, or other debris resulting from the cleanup of a spill, into or on any land or water, of any acute hazardous waste listed in in LAC 33:V.4901.B with the assigned hazard code of (H) or LAC 33:V.4901.E.

* * *

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 10:200 (March 1984), amended LR 10:496 (July 1984), LR 11:1139 (December 1985), LR 12:319 (May 1986), LR 13:84 (February 1987), LR 13:433 (August 1987), LR 13:651 (November 1987), LR 14:790, 791 (November 1988), LR 15:378 (May 1989), LR 15:737 (September 1989), LR 16:218, 220 (March 1990), LR 16:399 (May 1990), LR 16:614 (July 1990), LR 16:683 (August 1990), LR 17:362 (April 1991), LR 17:478 (May 1991), LR 18:723 (July 1992), LR 18:1375 (December 1992), repromulgated by the Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 19:626 (May 1993), amended LR 20:1000 (September 1994), LR 20:1109 (October 1994), LR 21:266 (March 1995), LR 21:944 (September 1995), LR 22:814 (September 1996), LR 23:564 (May 1997), amended by the Office of Waste Services, Hazardous Waste Division, LR 24:655 (April 1998), LR 24:1101 (June 1998), LR 24:1688 (September 1998), LR 25:433 (March 1999), repromulgated LR 25:853 (May 1999), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:269 (February 2000), LR 26:2465 (November 2000), LR 27:291 (March 2001), LR 27:708 (May 2001), LR 28:999 (May 2002), LR 28:1191 (June 2002), LR 29:318 (March 2003); amended by the Office of the Secretary, Legal Affairs Division, LR 31:2452 (October 2005), LR 31:3116 (December 2005), LR 32:606 (April 2006), LR 32:822 (May 2006), LR 33:1625 (August 2007), LR 33:2098 (October 2007), LR 34:71 (January 2008), LR 34:615 (April 2008), LR 34:1009 (June 2008), LR 34:1894 (September 2008), LR 34:2396 (November 2008), LR 36:1235 (June 2010), repromulgated LR 36:1535 (July 2010), amended LR 36:2554 (November 2010), LR 38:774, 781 (March 2012), repromulgated LR 38:1009 (April 2012), amended by the Office of the Secretary, Legal Division, LR 40:1338 (July 2014), LR 41:2600 (December 2015), LR 42:565 (April 2016), LR 42:2178 (December 2016), LR 43:1138 (June 2017), repromulgated by the Office of the Secretary, Legal Affairs and Criminal Investigation Division, LR 43:1531 (August 2017), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:898 (July 2020), LR 47:1851 (December 2021), amended by the Office of the Secretary, Legal Affairs Division LR 50:1457 (October 2024), LR 52:

Chapter 3.	General Conditions for Treatment, Storage, and Disposal Facility Permits

§305.	Scope of the Permit

A. — C.1.	…

2.	generators who accumulate hazardous waste on-site in compliance with all of the conditions for exemption provided in LAC 33:V.1009, 1011, 1013, and 1015, as applicable;

C.3. — H.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 10:200 (March 1984), amended LR 10:496 (July 1984), LR 13:84 (February 1987), LR 13:433 (August 1987), LR 16:220 (March 1990), LR 16:614 (July 1990), LR 17:658 (July 1991), LR 20:1000 (September 1994), LR 20:1109 (October 1994), LR 21:944 (September 1995), LR 23:567 (May 1997), amended by the Office of Waste Services, Hazardous Waste Division, LR 24:1105 (June 1998), LR 24:1690, 1759 (September 1998), LR 25:435 (March 1999), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 27:708 (May 2001), amended by the Office of the Secretary, Legal Affairs Division, LR 31:3116 (December 2005), LR 33:1625 (August 2007), LR 34:619 (April 2008), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:900 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Chapter 10.	Generators of Hazardous Waste

[Editor's Note:  Chapter 10 consolidates and reorganizes the requirements for generators formerly contained in LAC:V.108 and Chapter 11.]

Subchapter A.	General

§1003.	Purpose, Scope, and Applicability

A. — A.1.b.v.	…

vi.	LAC 33:V.1107 (The Manifest SystemRequirements);

A.1.b.vii. — F.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:901 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1007.	Generator Category Determination

A. — C.4.	…

| Table 1. Generator Categories Based on Quantity of Hazardous Waste Generated in a Calendar Month |  |  |  |
| --- | --- | --- | --- |
| Quantity of Acute Hazardous Waste Generated in a Calendar Month | Quantity of Nonacute Hazardous Waste Generated in a Calendar Month | Quantity of Residues from a Clean-up of Acute Hazardous Waste in a Calendar Month | Generator Category |
| * * * |  |  |  |
| Any Amount | Any Amount | Greater than 1001,000 kg (220 lbs) (>100 kg) | Large Quantity Generator |
| * * * |  |  |  |

D. — G.2.b.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.	HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:903 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1013.	Conditions for Exemption for Small Quantity Generators

A. — C.2.d.	…

i.	The small quantity generator shall record the inspections in an inspection log or summary. Records shall be kept for at least three years from the date of the inspection.  At a minimum, these records shall include:

(a).	the date and time of the inspection;

(b).	the name of the inspector;

(c).	a notation of observations made; and

(d).	the date and nature of any repairs or other remedial actions taken.

2.e. — 3.b.iv.	…

v.	the construction materials of, and the area immediately surrounding, discharge confinement structures (e.g., dikes) at least weekly to detect erosion or obvious signs of leakage (e.g., wet spots or dead vegetation). The generator shall remedy any deterioration or malfunction of equipment or structures, which the inspection reveals on a schedule, which ensures that the problem does not lead to an environmental or human health hazard. Where a hazard is imminent or has already occurred, remedial action shall be taken immediately.

3.c. — 4.	…

a.	comply with LAC 33:V.2801, 2803, 2804, 2805, 2807, and 2809.A and B;

4.b. — 5.b.	…

c.	provide an indication of the hazards of the contents in a conspicuous place (examples include, but are not limited to, the applicable hazardous waste characteristic(s) (i.e., ignitable, corrosive, reactive, toxic); hazardous communication consistent with the U.S. Department of Transportation requirements in 49 CFR part 172 subpart E (labeling) or subpart F (placarding); a hazard statement or pictogram consistent with the U.S. Occupational Safety and Health Administration Hazard Communication Standard in 29 CRFR 1910.1200; or a chemical hazard label consistent with the National Fire Protection Association Code 704);

5.d. — 6.a.i.	…

ii.	an indication of the hazards of the contents (examples include, but are not limited to, the applicable hazardous waste characteristic(s) (i.e., ignitable, corrosive, reactive, toxic); hazard communication consistent with the U.S. Department of Transportation requirements in 49 CFR part 172 subpart E (labeling) or subpart F (placarding); a hazard statement or pictogram consistent with the U.S. Occupational Safety and Health Administration Hazard Communication Standard in 29 CFR 1910.1200; or a chemical hazard label consistent with the National Fire Protection Association Code 704).

6.b. — 8.a.	…

b.	Required Equipment. All areas where hazardous waste is either generated or accumulated shall be equipped with the items in Clauses 1013.C.8.b.i-iv of this Section, unless none of the hazards posed by waste handled at the facility could require a particular kind of equipment specified below or the actual waste generation or accumulation area does not lend itself for safety reasons to have a particular kind of equipment specified below. A small quantity generator may determine the most appropriate locations to locate equipment necessary to prepare for and respond to emergencies. The required equipment consists of:

C.8.b.i. — G.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:906 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1015.	Conditions for Exemption for Large Quantity Generators

A. — B.1.	…

a.	Air Emission Standards. The applicable requirements of LAC 33:V.Chapter 43.Subchapters Q, R, and V;

B.1.b. — e.	…

i.	The large quantity generator shall record the inspections in an inspection log or summary. The generator shall keep these records for at least three years from the date of the inspection. At a minimum, these records shall include:

(a).	the date and time of the inspection;

(b).	the name of the inspector;

(c).	a notation of observations made; and

(d).	the date and nature of any repairs or other remedial actions taken.

1.f. — 8.a.	…

i.	The large quantity generator shall notify the Office of Environmental Services following the procedures in Subparagraph B.8.b of this Paragraph in order to meet the closure performance standards of Clause B.8.c.i of this Paragraph for container storage, tank systems, and containment buildings or Clause B.8.c.ii of this Paragraph for drip pads. If the central accumulation area is subsequently reopened, the large quantity generator shall update the notice in the operating record.

a.ii. — c.i.(b). 	…

(c).	If the large quantity generator demonstrates that either any contaminated soils and wastes cannot be practicably removed or decontaminated as required in Division B.8.c.i.(a).(ii) of this Paragraph, or that the remaining contaminant levels are not protective of human health and the environment as demonstrated by the confirmatory sampling and analytical results specified in Subdivision B.8.b.ii.(c).(ii).[c] and [d] of this Paragraph, or through the use of RECAP and remedial activities under Subparagraph B.8.f of this Paragraph or LAC 33:V.2809.B.2, then the central accumulation area is considered to be a landfill. The large quantity generator shall then close the central accumulation area and perform post-closure care in accordance with the closure and post-closure care requirements that apply to landfills (LAC 33:V.4501.B and D). In addition, for the purposes of closure, post-closure, and financial responsibility, such a central accumulation area is then considered to be a landfill, and the large quantity generator shall meet all of the requirements for landfills specified in LAC 33:V.Chapter 43.Subchapters F and G.

c.ii. — i.	…

j.	Closure Guidance. The large quantity generator shouldshall review all guidance that may be issued by the department and/or posted on its website including, but not limited to, guidance on confirmatory sampling for aboveground structures and environmental media. The purpose of such guidance is to ensure best management practices, promote consistency, and produce technically defensible closures. Any such guidance issued by the department is not regulation and shall not substitute for the requirements of Paragraph B.8 of this Subparagraph. Thus, any guidance does not impose any new requirements. The department shall retain discretion to use approaches on a case-by-case basis that differ from such guidance where appropriate. The department will basemake decisions regarding closure activities required by Subparagraph B.8 of this Paragraph in accordance with the Act and regulations as applied to the specific facts of the closure. Whether or not the recommendations in any guidance are appropriate in a given situation will depend on site-specific circumstances.

B.8.k. — G.	…

1.	The large quantity generator shall notify the Office of Environmental Services at least 30 calendar days prior to receiving the first shipment from a very small quantity generator(s) using the department’s Notification of Hazardous Waste Activity Form (HW-1) that:

a.	identifies on the formthe EPA identification number(s), if applicable, the name(s) and site address(es) for the very small quantity generator(s) as well as the contact name and business telephone numbercontact information for a contact person for the very small quantity generator(s); and

b.	submits an update of the department’s Notification of Hazardous Waste Activity Form (HW-1) within 30 calendar days after a change in the name or site address for the very small quantity generator.

G.2. — H.2.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:911 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1017.	EPA Identification Numbers and Notification of Hazardous Waste Activities for Generators

A.	All generators (i.e., very small quantity generators, small quantity generators and large quantity generators) shall obtain an active EPA identification number by notifying the Office of Environmental Services using the Notification of Hazardous Waste Activity Form (HW-1) within 14 days after first generating any hazardous waste at the location specified in the notification. The assignment of an active EPA identification number shall serve as proof of this notification to the department by the generator. However, as EPA identification numbers are site-specific, if a generator moves to another location, the generator shall obtain a new EPA identification number for the facility. A generator shall notify the Office of Environmental Services within seven days if any information submitted in the notification of hazardous waste activity changes. As stated in LAC 33:V.105.A.910, failure to submit a timely and complete Notification of Hazardous Waste Activity Form (HW-1), obtain an active EPA identification number or notify the department of changes to the notification shall constitute a violation of these regulations and subject the applicant to enforcement action up to and including the assessment of civil penalties.

B. — D.1.	…

a.	an even number, including zero, shall submit notification by April 15, 2021, and every four years thereafter; or

b.	…

2.	A large quantity generator shall renotify the Office of Environmental Services by March 1 of each even-numbered year thereafter using the department’s Notification of Hazardous Waste Activity Form (HW-1) or other forms approved by the department. A large quantity generator may submit this renotification as part of its annual report required under LAC 33:V.1021.

E. — F.	…

G.	Generators who cease hazardous waste activities or move to another location shall notify the Office of Environmental Services within 30 days using the department’s Notification of Hazardous Waste Activity Form (HW-1) or other forms approved by the department in accordance with LAC 33:V.105.A.5.bshall comply with the notification requirements in LAC 33:V.105.A.8.

H.	Generators who move to another location shall comply with the notification requirements in LAC 33:V.105.A.9.

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:919 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Subchapter B.	Recordkeeping and Reporting for Small Quantity Generators and Large Quantity Generators

§1019.	Recordkeeping

A. — D.	…

E.	All records, including plans, required under Subchapter B must be furnished upon request, and made available at all reasonable times for inspection, by any officer, employee, or representative who is duly designated by the administrative authority.

AUTHORITY NOTE: Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE: Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:920 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1023.	Exception Reporting

A. — C.	…

D.	For rejected shipments of hazardous waste or container residues contained in nonempty containers that are forwarded to an alternate facility by a designated facility using a new manifest, following the procedures of LAC 33:V.1516.C.5.a.i-vi, the generator shall comply with the requirements of Subsections A or C of this Section, as applicable, for the shipment forwarding the material from the designated facility to the alternate facility instead of for the shipment from the generator to the designated facility. For purposes of Subsections A- or C of this Section for a shipment forwarding such waste to an alternate facility by a designated facility, the following conditions shall apply.

1. — 2.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:919 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Subchapter C.	Alternative Standards for Episodic Generation

§1031.	Definitions for this Subchapter

A.	…

Episodic Event—an activity or activities, either planned or unplanned, that does not normally occur during generator operations, resulting in an increase in the generation of hazardous wastes that exceeds the calendar month quantity limits for the generator’s usual category. An episodic event shall not last more than 60 calendar days, beginning with the initial generation and accumulation of the episodic-generated hazardous waste, regardless of whetherwhen the generator has determined the waste is hazardous, and concluding with the episodic-generated hazardous waste being sent to a designated facility.

* * *

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:921 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1033.	Conditions for Generators Managing Hazardous Waste from an Episodic Event

A. — A.1.	…

2.	Notification. The very small quantity generator shall notify the Office of Environmental Services no later than 30 calendar days prior to initiating a planned episodic event (i.e., commencing the generation and accumulation of the episodic-generated hazardous waste) using the department’s Notification of Hazardous Waste Activity Form (HW-1). In the event of an unplanned episodic event, the generator shall notify the Office of Environmental Services Single Point of Contact (SPOC) within 72 hours of initiating the unplanned event (i.e., commencing the generation and accumulation of the episodic-generated hazardous waste) via phone, email, or faxonline incident reporting, as specified in LAC 33:I.3923, and subsequently submit the department’s Notification of Hazardous Waste Activity Form (HW-1) to the Office of Environmental Services within 15 days of initiating the unplanned event. The generator shall include the start date and end date of the episodic event, the reason(s) for the event, types and estimated quantities of hazardous waste expected to be generated as a result of the episodic event, and shall identify a facility contact and emergency coordinator with 24-hour telephone access to discuss the notification submittal or respond to an emergency in compliance with LAC 33:V.1013.C.9.a.

3. — 7.e.	…

f.	an approval letter from the administrative authorityOffice of Environmental Services if the generator petitioned to conduct one additional episodic event per calendar year.

8.	Withdrawal of Notification

a.	The very small quantity generator may withdraw the notification for the episodic event by submitting a letter to the Office of Environmental Services no later than 120 calendar days after submitting the initial notification (HW-1 Form). The letter shall specify the reason for the withdrawal and shall be signed by the owner/operator or a duly authorized representative. Reasons for the withdrawal may include the:

i.	episodic event did not occur;

ii.	waste was determined not to be a hazardous waste in accordance with LAC 33:V.1005; or

iii.	total amount (combined episodic and nonepisodic) of hazardous waste generated in a calendar month did not exceed the very small quantity generator category limit in accordance with LAC 33:V.1007.

B. — B.1.	…

2.	Notification. The small quantity generator shall notify the Office of Environmental Services no later than 30 calendar days prior to initiating a planned episodic event (i.e., commencing the generation and accumulation of the episodic-generated hazardous waste) using the department’s Notification of Hazardous Waste Activity Form (HW-1). In the event of an unplanned episodic event, the small quantity generator shall notify the Office of Environmental ServicesSPOC within 72 hours of initiating the unplanned event (i.e., commencing the generation and accumulation of the episodic-generated hazardous waste) via phone, email, or faxonline incident reporting as specified in LAC 33:I.3923, and subsequently submit the department’s Notification of Hazardous Waste Activity Form (HW-1) to the Office of Environmental Services within 15 days of initiating the unplanned event. The small quantity generator shall include the start date and end date of the episodic event and the reason(s) for the event, types and estimated quantities of hazardous waste expected to be generated as a result of the episodic event, and identify a facility contact and emergency coordinator with 24-hour telephone access to discuss the notification submittal or respond to emergency.

3. — 6.e.	…

f.	an approval letter from the administrative authorityOffice of Environmental Services if the generator petitioned to conduct one additional episodic event per calendar year.

7.	Withdrawal of Notification

a.	The small quantity generator may withdraw the notification for the episodic event by submitting a letter to the Office of Environmental Services no later than 120 calendar days after submitting the initial notification (HW-1 Form). The letter shall specify the reason for the withdrawal and shall be signed by the owner/operator or a duly authorized representative. Reasons for the withdrawal may include the:

i.	episodic event did not occur;

ii.	waste was determined not to be a hazardous waste in accordance with LAC 33:V.1005; or

iii.	total amount (combined episodic and nonepisodic) of hazardous waste generated in a calendar month did not exceed the small quantity generator category limit in accordance with LAC 33:V.1007.

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:921 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§1035.	Petition to Manage One Additional Episodic Event per Calendar Year

A.	A generator may petition the administrative authorityOffice of Environmental Services for a second episodic event in a calendar year without impacting its generator category under the following conditions. If a very small quantity generator or small quantity generator has already held:

1.	a planned episodic event in a calendar year, the generator may petition the administrative authorityOffice of Environmental Services for an additional unplanned episodic event in that calendar year within 72 hours of initiating the unplanned event; or

2.	an unplanned episodic event in a calendar year, the generator may petition the administrative authorityOffice of Environmental Services for an additional planned episodic event in that calendar year. (NOTE: The petition shall be submitted and approved prior to notification of the planned event [i.e., at least 30 calendar days prior to commencement].)

B. — B.5.	…

C.	The petition shall be made to the administrative authorityOffice of Environmental Services in writing, either on paper or electronically.

D.	If the petition is approved by the administrative authorityOffice of Environmental Services, the generator shall comply with Section 1033 of this Subchapter when managing the hazardous waste from the second approved episodic event including notifying the Office of Environmental Services using the department’s Notification of Hazardous Waste Activity Form (HW-1). A copy of the written approval of the petition shall accompany the HW-1 notification.

E.	...

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:923 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Subchapter D.	Preparedness, Prevention and Emergency Procedures for Large Quantity Generators

§1045.	Access to Communication or Alarm Systems

A.	Whenever hazardous waste is being poured, mixed, spread, or otherwise handled, all personnel involved in the operation shall have immediate access (e.g., direct or unimpeded access) to an internal alarm or emergency communication device, either directly or through visual or voice contact with another employee, unless such a device is not required under Section 1041 of this Subchapter.

B.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:924 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Chapter 22.	Prohibitions on Land Disposal

Subchapter A.	Land Disposal Restrictions

§2201.	Purpose, Scope, and Applicability

A. — I.3.	…

4.	waste generated by very small quantity generator, as defined in LAC 33:V.1009109;

5. — 5.f.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 15:378 (May 1989), amended LR 16:398 (May 1990), LR 16:1057 (December 1990), LR 17:658 (July 1991), LR 18:723 (July 1992), LR 21:266 (March 1995), LR 22:22 (January 1996), LR 23:568 (May 1997), amended by the Office of Waste Services, Hazardous Waste Division, LR 24:300 (February 1998), LR 24:666 (April 1998), LR 24:1107 (June 1998), LR 24:1724 (September 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 25:1799 (October 1999), LR 27:711 (May 2001), amended by the Office of the Secretary, Legal Affairs Division, LR 31:3117 (December 2005), amended by the Office of the Secretary, Legal Division, LR 43:1142 (June 2017), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:936 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Subchapter B.	Hazardous Waste Injection Restrictions

§2249.	Purpose, Scope, and Applicability

A. — C.2.	…

3.	if the waste is generated by a very small quantity generator, as defined in LAC 33:V.1009109.

D. — D.2.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 22:22 (January 1996), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 25:1800 (October 1999), LR 27:712 (May 2001), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:938 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Chapter 30.	Hazardous Waste Burned in Boilers and Industrial Furnaces

§3001.	Applicability

A. — C.2.	…

3.	hazardous wastes that are exempt from regulation under LAC 33:V.105.D and 4105.A.1.c-d.iii, and hazardous wastes that are subject to the special requirements for very small quantity generators under LAC 33:V.1007 and 1009; and

C.4. — H.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2180 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 18:1375 (December 1992), amended LR 21:266 (March 1995), LR 21:944 (September 1995), LR 22:821, 835 (September 1996), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 25:1466 (August 1999), LR 27:297 (March 2001), LR 27:712 (May 2001), LR 29:323 (March 2003), amended by the Office of the Secretary, Legal Affairs Division, LR 32:607 (April 2006), LR 34:628 (April 2008), LR 34:1014 (June 2008), amended by the Office of the Secretary, Legal Division, LR 43:1145 (June 2017), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:939 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Chapter 51.	Fee Schedules

§5121.	Generators and Transporters of Hazardous Waste

A. — B.1.b.	…

2.	90-day Storage Extension. Application for 30-day Extension of Accumulation Time Limit in LAC 33:V.1013.E and LAC 33:V.1015.C. All requests for extension of accumulation time limit shall be accompanied by a $500 application fee.

C. — C.3.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2014 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Hazardous Waste Division, LR 10:200 (March 1984), amended LR 11:533 (May 1985), LR 12:676 (October 1986), LR 14:621 (September 1988), amended by the Office of the Secretary, Legal Division, LR 43:944 (May 2017), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 46:949 (July 2020), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

Part VII.  Solid Waste

Subpart 1.  Solid Waste Regulations

Chapter 1.	General Provisions and Definitions

§115.	Definitions

A.	...

* * *

Hazardous Waste Determination—the process performed in accordance with LAC 33:V.11031005 and LAC 33:VII.119.

* * *

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq. and in particular R.S. 30:2154.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 22:279 (April 1996), amended by the Office of Waste Services, Solid Waste Division, LR 23:1145 (September 1997), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2514, 2609 (November 2000), amended by the Office of Environmental Assessment, LR 31:1576 (July 2005), amended by the Office of the Secretary, Legal Affairs Division, LR 33:1019 (June 2007), LR 34:1023 (June 2008), LR 34:1399 (July 2008), LR 37:1563 (June 2011), LR 37:3233 (November 2011), LR 38:46 (January 2012), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 45:234 (February 2019), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§119.	Hazardous Waste Determination

A.	A person who generates a solid waste, as defined in LAC 33:V.109, shall determine if that waste is a hazardous waste in accordance with LAC 33:V.1005.A.

B.	The hazardous waste determination for each solid waste shall be made at the point of waste generation, before any dilution, mixing, or other alteration of the waste occurs, and/or at any time in the course of its management that it has, may have, or changed its properties as a result of exposure to the environment or other factors that may change the properties of the waste in accordance with LAC 33:V.1005.B.

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Office of the Secretary, Legal Affairs Division, LR 52:

Chapter 3.	Scope and Mandatory Provisions of the Program

§315.	Mandatory Provisions

A. — O.	…

P.	Generators shall conduct a hazardous waste determination, as outlined in LAC 33:VII.119 on all generated wastes.

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 19:1143 (September 1993), LR 19:1315 (October 1993), repromulgated LR 19:1421 (November 1993), amended LR 22:279 (April 1996), amended by the Office of Waste Services, Solid Waste Division, LR 23:954 (August 1997), LR 23:1145 (September 1997), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2516 (November 2000), LR 30:1675 (August 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2487 (October 2005), LR 33:1030 (June 2007), LR 34:1400 (July 2008), LR 36:1240 (June 2010), LR 37:3235 (November 2011) repromulgated LR 37:3508 (December 2011), LR 52:

Chapter 7.	Solid Waste Standards

Subchapter A.	Landfills, Surface Impoundments, Landfarms

§711.	Standards Governing Landfills (Type I and II)

A. — D.3.c.ii.	…

d.	Waste Characterization. The permit holder shall review and maintain the hazardous waste determination performed by the generator in accordance with LAC 33:V.11031005 for all solid waste prior to acceptance. Every year thereafter, the permit holder shall require the generator to submit either a written certification that the waste being sent to the permit holder remains unchanged or a new waste characterization. All characterizations and certification records shall be maintained on- site for a period of three years.

D.3.e. — F.3.d.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended LR 19:1143 (September 1993), repromulgated LR 19:1316 (October 1993), amended by the Office of the Secretary, LR 24:2251 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2523 (November 2000), repromulgated LR 27:704 (May 2001), amended LR 30:1676 (August 2004), amended by the Office of Environmental Assessment, LR 30:2024 (September 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2492 (October 2005), LR 33:1047 (June 2007), LR 33:2145 (October 2007), LR 34:1901 (September 2008), LR 37:1564 (June 2011), LR 37:3248 (November 2011), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§713.	Standards Governing Surface Impoundments (Type I and II)

A. — D.3.d.	…

e.	Waste Characterization. The permit holder shall review and maintain the hazardous waste determination performed by the generator in accordance with LAC 33:V.11031005 for all solid waste prior to acceptance. Every year thereafter, the permit holder shall require the generator to submit either a written certification that the waste being sent to the permit holder remains unchanged or a new waste characterization. All characterizations and certification records shall be maintained on-site for a period of three years.

D.4 — F.2.b.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), repromulgated LR 19:1316 (October 1993), amended by the Office of the Secretary, LR 24:2251 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2524 (November 2000), repromulgated LR 27:704 (May 2001), amended LR 30:1676 (August 2004), amended by the Office of Environmental Assessment, LR 30:2025 (September 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2493 (October 2005), LR 33:1053 (June 2007), LR 33:2146 (October 2007), LR 36:1241 (June 2010), LR 37:1564 (June 2011), LR 37:3250 (November 2011), repromulgated LR 37:3511 (December 2011), amended by the Office of the Secretary, Legal Affairs and Criminal Investigations Division, LR 45:235 (February 2019), amended by the Office of the Secretary, Legal Affairs Division, LR 52:

§715.	Standards Governing Landfarms (Type I and II)

A. — D.3.j.	…

k.	Waste Characterization. The permit holder shall review and maintain the hazardous waste determination performed by the generator in accordance with LAC 33:V.11031005 for all solid waste prior to acceptance. Every year thereafter, the permit holder shall require the generator to submit either a written certification that the waste being sent to the permit holder remains unchanged or a new waste characterization. All characterizations and certification records shall be maintained on-site for a period of three years.

D.4 – F.3.b.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), repromulgated LR 19:1316 (October 1993), amended by the Office of the Secretary, LR 24:2251 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2525 (November 2000), repromulgated LR 27:704 (May 2001), amended LR 30:1676 (August 2004), amended by the Office of Environmental Assessment, LR 30:2025 (September 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2493 (October 2005), LR 33:1058 (June 2007), LR 33:2147 (October 2007), LR 35:1880 (September 2009), LR 37:1565 (June 2011), LR 37:3251 (November 2011), repromulgated LR 37:3511 (December 2011), LR 52:

Subchapter B.	Solid Waste Processors

§717.	Standards Governing All Type I-A and II-A Solid Waste Processors

A. — G.3.	…

a.	Waste Characterization. The permit holder shall review and maintain the hazardous waste determination performed by the generator in accordance with LAC 33:V.11031005 for all solid waste prior to acceptance. Every year thereafter, the permit holder shall require the generator to submit either a written certification that the waste being sent to the permit holder remains unchanged or a new waste characterization. All characterizations and certification records shall be maintained on-site for a period of three years.

G.3.b. — I.3.	…

AUTHORITY NOTE:	Promulgated in accordance with R.S. 30:2001 et seq.

HISTORICAL NOTE:	Promulgated by the Department of Environmental Quality, Office of Solid and Hazardous Waste, Solid Waste Division, LR 19:187 (February 1993), amended by the Office of the Secretary, LR 24:2252 (December 1998), amended by the Office of Environmental Assessment, Environmental Planning Division, LR 26:2526, 2610 (November 2000), repromulgated LR 27:704 (May 2001), amended by the Office of Environmental Assessment, LR 30:2025 (September 2004), amended by the Office of the Secretary, Legal Affairs Division, LR 31:2494 (October 2005), LR 33:1061 (June 2007), LR 33:2148 (October 2007), LR 34:613 (April 2008), LR 35:926 (May 2009), LR 37:1566 (June 2011), LR 37:3252 (November 2011), amended by the Office of the Secretary, Legal Division, LR 40:295 (February 2014), LR 52: